Note:	Release 1.2.3 is currently in a Beta release state.  All information and code associated with this release is subject to change.

This document describes the changes associated with each release of the Silicon Image SATARaid5 driver.

#### Version 1.2.3

Product Versions

Version 1.2.3 contains the following versions of the SATARaid5 Driver, BIOS and Manager (GUI) for the listed supported Silicon Image products.  An asterisk (*) indicates a change from the previous release.

| ###### Product | Driver | BIOS | Manager |
| --- | --- | --- | --- |
| Sil3114 | 1.2.3.1* | r5200.bin | 1.0.1.0 |
| Sil3124 | 1.2.3.1* | r6206.bin | 1.0.1.0 |
| Sil3132 | 1.2.3.1* | r7105.bin | 1.0.1.0 |

Fixes

#1	WindowsTM cannot hibernate if no drive is present on channel 0.

An error in the driver was providing unexpected status to the operating system when a reset occurred during hibernation and there was no drive present on channel 0 of the controller.  The bug was fixed and tested.

#2	Possible system hang after repeated intermittent errors

If an intermittent hardware error caused a drive to be marked as removed and reinserted in a very short time period, the driver could hang resulting in no more data being sent to or read from the attached disks.  The bug was fixed and tested.

#3	Possible data loss during unexpected power downs with an active rebuild.

If an unexpected power down occurred while a rebuild operation was in progress, it is possible that data being written to the volume was lost.  The bug was fixed and tested.

#4	Incorrect display of volume status.  Following a restart, it was possible for the GUI to display a status of green (normal) during a rebuild instead of the appropriate yellow (degraded).  The bug was fixed and tested.

#### Version 1.2.2

Product Versions

Version 1.2.2 contains the following versions of the SATARaid5 Driver, BIOS and Manager (GUI) for the listed supported Silicon Image products.  An asterisk (*) indicates a change from the previous release.

| ###### Product | Driver | BIOS | Manager |
| --- | --- | --- | --- |
| Sil3114 | 1.2.0.0 | r5200.bin | 1.0.1.0* |
| Sil3124 | 1.2.0.0 | r6206.bin | 1.0.1.0* |
| Sil3132 | 1.2.1.0 | r7105.bin | 1.0.1.0* |

Enhancements

Version 1.0.1.0 of the SATARaid5 Manager now supports multiple controller cards.  Upon starting the manager, the left pane of the window will display all detected Silicon Image RAID compatible cards as shown in the following diagram.

To manage a particular controller, simply click on the desired controller in the left hand pane.